### Prerequisites

- Hardware: M5Stack Core2, APDS-9960 Unit (connected to Port A - Red), PC/Mac.

- Arduino Libraries: Install these via Library Manager:

  - M5Unified

  - Adafruit APDS9960 Library

  - ArduinoJson

### Part 1: Host System Setup (InfluxDB & Grafana)

We will install the software natively (without Docker) to act as the backend.

#### A. macOS (via Homebrew)

- Install & Start InfluxDB:

Bash

brew update
brew install influxdb
# Start the service
influxd

- Install & Start Grafana:

Bash

brew install grafana
# Start the service
brew services start grafana

#### B. Windows (Manual Install)

- InfluxDB:

  - Download the InfluxDB v2.x binary (zip) from the InfluxData downloads page.

  - Extract the folder.

  - Open PowerShell, navigate to that folder, and run ./influxd.exe.

- Grafana:

  - Download the Grafana Installer for Windows from Grafana Labs.

  - Run the installer. Grafana will automatically start as a Windows Service.

#### C. Configuration (Both Systems)

- Setup InfluxDB:

  - Open a browser to http://localhost:8086.

  - Click "Get Started". Set up a user (e.g., admin), password, Organization (e.g., UNLV_Lab), and Bucket (e.g., sensor_data).

  - Crucial: Copy the API Token presented at the end. You need this for the Python script.

### Part 2: Arduino Firmware (M5Core2)

This script reads the sensor and pushes JSON data over TCP.

- Note: The APDS-9960 on M5Core2 Port A uses pins 32 (SDA) and 33 (SCL).

C++

#include <M5Unified.h>
#include <Wire.h>
#include <WiFi.h>
#include <ArduinoJson.h>
#include <Adafruit_APDS9960.h>

// --- CONFIGURATION ---
const char* ssid     = "YOUR_WIFI_SSID";
const char* password = "YOUR_WIFI_PASSWORD";
const char* host     = "192.168.1.100"; // IP address of your PC/Mac
const int   port     = 65432;           // Same port as Python script

Adafruit_APDS9960 apds;
WiFiClient client;

void setup() {
  auto cfg = M5.config();

M5.begin(cfg);
  
  M5.Display.setTextSize(2);
  M5.Display.println("Initializing...");

  // Initialize I2C for Port A (Core2 uses 32, 33)
  Wire.begin(32, 33);

  if(!apds.begin()){
    M5.Display.println("Failed to find APDS9960 sensor!");
    while(1) delay(100);
  }
  apds.enableColor(true); // Enable color sensing

  // Connect to WiFi
  M5.Display.print("Connecting to WiFi");
  WiFi.begin(ssid, password);
  while (WiFi.status() != WL_CONNECTED) {
    delay(500);
    M5.Display.print(".");
  }
  M5.Display.println("\nWiFi Connected!");
  M5.Display.println(WiFi.localIP());
}

void loop() {
  M5.update();
  
  // 1. Read Sensor Data
  uint16_t r, g, b, c;
  
  // Wait for color data to be ready
  while(!apds.colorDataReady()) {
    delay(5);
  }
  apds.getColorData(&r, &g, &b, &c);

  // 2. Create JSON Document
  StaticJsonDocument<200> doc;
  doc["location"] = "Lab_Station_1";
  doc["timestamp"] = millis(); // Relative time (Python will handle real timestamp)
  doc["red"] = r;
  doc["green"] = g;
  doc["blue"] = b;
  doc["clear"] = c;

  String jsonString;
  serializeJson(doc, jsonString);

  // 3. Send via TCP
  if (client.connect(host, port)) {
    client.println(jsonString);
    client.stop(); // Close connection after sending (Short-lived connection)
    M5.Display.fillScreen(BLACK);
    M5.Display.setCursor(0,0);
    M5.Display.printf("Sent:\nR:%d G:%d B:%d", r, g, b);
  } else {
    M5.Display.fillScreen(RED);
    M5.Display.setCursor(0,0);
    M5.Display.println("Connection Failed");
  }

  delay(1000); // 1 Hz sample rate
}

### Part 3: Python Host Receiver

This script listens for the M5Stack's connection and writes data to InfluxDB.

Install Dependencies:

Bash

pip install influxdb-client

Python Script (bridge.py):

Python

import socket
import json
import datetime
from influxdb_client import InfluxDBClient, Point
from influxdb_client.client.write_api import SYNCHRONOUS

# --- CONFIGURATION ---
HOST_IP = '0.0.0.0' # Listen on all interfaces
PORT = 65432

# InfluxDB Settings (Match what you set up in UI)
INFLUX_URL = "http://localhost:8086"
INFLUX_TOKEN = "YOUR_ADMIN_TOKEN_HERE"
INFLUX_ORG = "UNLV_Lab"
INFLUX_BUCKET = "sensor_data"

def write_to_influx(data_dict):
    try:
        client = InfluxDBClient(url=INFLUX_URL, token=INFLUX_TOKEN, org=INFLUX_ORG)
        write_api = client.write_api(write_options=SYNCHRONOUS)

        # Create Point
        p = Point("light_measurement") \
            .tag("location", data_dict.get("location", "unknown")) \
            .field("red", int(data_dict["red"])) \
            .field("green", int(data_dict["green"])) \
            .field("blue", int(data_dict["blue"])) \
            .field("clear", int(data_dict["clear"])) \
            .time(datetime.datetime.utcnow())

        write_api.write(bucket=INFLUX_BUCKET, org=INFLUX_ORG, record=p)
        print(f"Written: R={data_dict['red']} G={data_dict['green']} B={data_dict['blue']}")
        
    except Exception as e:
        print(f"Database Error: {e}")

def start_server():
    server_socket = socket.socket(socket.AF_INET, socket.SOCK_STREAM)
    server_socket.bind((HOST_IP, PORT))
    server_socket.listen(5)
    print(f"Listening on {PORT}...")

    while True:
        client_socket, addr = server_socket.accept()
        # Read data (Buffer size 1024 bytes)
        data = client_socket.recv(1024)
        
        if data:
            try:
                # Decode and Parse JSON
                json_str = data.decode('utf-8').strip()
                data_dict = json.loads(json_str)
                write_to_influx(data_dict)
            except json.JSONDecodeError:
                print("Received invalid JSON")
            except Exception as e:
                print(f"Error: {e}")
        
        client_socket.close()

if __name__ == "__main__":
    start_server()

### Part 4: Visualization in Grafana

- Login: Go to http://localhost:3000 (Default user/pass: admin / admin).

- Add Data Source:

  - Configuration (Gear Icon) -> Data Sources -> Add data source.

  - Select InfluxDB.

  - Query Language: Select Flux.

  - URL: http://localhost:8086.

  - Auth: Turn off "Basic Auth".

  - InfluxDB Details:

    - Organization: UNLV_Lab

    - Token: YOUR_ADMIN_TOKEN

    - Default Bucket: sensor_data

  - Click Save & Test.

- Create Dashboard:

  - Click + -> Dashboard.

  - Add Visualization. Select your InfluxDB source.

  - Query (Flux):

Code snippet

from(bucket: "sensor_data")
  |> range(start: -1h)
  |> filter(fn: (r) => r["_measurement"] == "light_measurement")
  |> filter(fn: (r) => r["_field"] == "red" or r["_field"] == "blue" or r["_field"] == "green")
  |> aggregateWindow(every: 1s, fn: mean, createEmpty: false)
  |> yield(name: "mean")

  - Click Apply. You should now see a live graph of your RGB values.

### Part 1: Hardware Setup (RPi Zero W2)

Wiring the APDS-9960 to RPi:

- VCC $\rightarrow$ Pin 1 (3.3V)

- GND $\rightarrow$ Pin 6 (GND)

- SDA $\rightarrow$ Pin 3 (SDA / GPIO 2)

- SCL $\rightarrow$ Pin 5 (SCL / GPIO 3)

Software Configuration:

- Enable I2C:

Bash

sudo raspi-config
# Navigate to: Interface Options -> I2C -> Enable
sudo reboot

- Install Python Libraries:

Bash

sudo apt-get update
sudo apt-get install i2c-tools python3-pip
# Install the APDS9960 library and smbus
pip3 install apds9960 smbus2

### Part 2: Raspberry Pi Sender Script

Save this as rpi_sender.py on your Raspberry Pi. This script reads the sensor and streams JSON data.

Python

import time
import socket
import json
from smbus2 import SMBus
from apds9960.const import *
from apds9960 import APDS9960

# --- CONFIGURATION ---
HOST_IP = '192.168.1.100'  # REPLACE with your Host PC IP Address
PORT = 65432               # Must match the host script
LOCATION_ID = "Lab_RPi_Zero"

def setup_sensor():

bus = SMBus(1)
    apds = APDS9960(bus)
    apds.enableLightSensor()
    return apds

def main():
    # 1. Setup Sensor
    try:
        apds = setup_sensor()
        print("Sensor initialized.")
    except Exception as e:
        print(f"Error initializing sensor: {e}")
        return

    # 2. Connect to Host
    while True:
        try:
            print(f"Connecting to {HOST_IP}:{PORT}...")
            client_socket = socket.socket(socket.AF_INET, socket.SOCK_STREAM)
            client_socket.connect((HOST_IP, PORT))
            print("Connected!")
            break
        except ConnectionRefusedError:
            print("Host not found. Retrying in 5 seconds...")
            time.sleep(5)

    # 3. Stream Data
    try:
        while True:
            # Read RGB and Clear light
            r = apds.readRedLight()
            g = apds.readGreenLight()
            b = apds.readBlueLight()
            c = apds.readAmbientLight()

            # Create Payload
            payload = {
                "location": LOCATION_ID,
                "timestamp": time.time(), # Unix timestamp
                "red": r,
                "green": g,
                "blue": b,
                "clear": c
            }

            # Send JSON with newline delimiter
            json_str = json.dumps(payload) + "\n"
            client_socket.sendall(json_str.encode('utf-8'))
            
            print(f"Sent: R:{r} G:{g} B:{b} C:{c}")
            time.sleep(1) # 1Hz Sample Rate

    except KeyboardInterrupt:
        print("\nStopping...")
    except Exception as e:
        print(f"Connection Error: {e}")
    finally:
        client_socket.close()

if __name__ == "__main__":
    main()

### Part 3: Host System Setup (Mac & Windows)

#### A. Install InfluxDB & Grafana

For macOS (Homebrew):

Bash

# 1. InfluxDB
brew install influxdb
influxd # Start InfluxDB in a new terminal

# 2. Grafana
brew install grafana
brew services start grafana

For Windows:

- InfluxDB: Download the v2.x binary from InfluxData, extract it, and run influxd.exe via PowerShell.

- Grafana: Download the Windows Installer from Grafana Labs and run it.

#### B. Configure Database

- Go to http://localhost:8086 in your browser.

- Click "Get Started" -> Set Username/Password.

- Organization Name: UNLV_Lab

- Bucket Name: light_data

- Copy the API Token shown on the screen.

### Part 4: Host Receiver Script

Save this as host_receiver.py on your PC/Mac.

Install Dependencies:

Bash

pip install influxdb-client

The Code:

Python

import socket
import json
from influxdb_client import InfluxDBClient, Point
from influxdb_client.client.write_api import SYNCHRONOUS
from datetime import datetime

# --- CONFIGURATION ---
HOST_IP = '0.0.0.0' # Listen on all interfaces
PORT = 65432

# InfluxDB Config
URL = "http://localhost:8086"
TOKEN = "YOUR_API_TOKEN_HERE" # Paste token from Step 3B
ORG = "UNLV_Lab"
BUCKET = "light_data"

def write_to_db(write_api, data):
    """Writes the JSON data to InfluxDB"""
    # Convert Unix timestamp to datetime object
    dt_object = datetime.fromtimestamp(data['timestamp'])
    
    point = Point("apds9960_measurement") \
        .tag("location", data['location']) \
        .field("red", int(data['red'])) \
        .field("green", int(data['green'])) \
        .field("blue", int(data['blue'])) \
        .field("clear", int(data['clear'])) \
        .time(dt_object)
    
    write_api.write(bucket=BUCKET, org=ORG, record=point)

def main():
    # Setup InfluxDB Client
    client = InfluxDBClient(url=URL, token=TOKEN, org=ORG)
    write_api = client.write_api(write_options=SYNCHRONOUS)

    # Setup Socket Server
    server = socket.socket(socket.AF_INET, socket.SOCK_STREAM)
    server.bind((HOST_IP, PORT))
    server.listen(1)
    
    print(f"Listening for RPi on port {PORT}...")

    conn, addr = server.accept()
    with conn:
        print(f"Connected by {addr}")
        buffer = ""
        
        while True:
            chunk = conn.recv(1024)
            if not chunk: break
            
            buffer += chunk.decode('utf-8')
            
            # Handle newline delimited JSON
            while "\n" in buffer:
                line, buffer = buffer.split("\n", 1)
                if not line: continue
                
                try:
                    data = json.loads(line)
                    write_to_db(write_api, data)
                    print(f"Logged: R={data['red']} G={data['green']} B={data['blue']}")
                except json.JSONDecodeError:
                    print("Error decoding JSON")
                except Exception as e:
                    print(f"Error: {e}")

if __name__ == "__main__":
    main()

### Part 5: Grafana Visualization

- Login: Open http://localhost:3000 (User: admin / Pass: admin).

- Connect Data Source:

  - Go to Connections -> Data Sources -> Add new.

  - Select InfluxDB.

  - Query Language: Flux.

  - URL: http://localhost:8086.

  - Auth: Uncheck "Basic Auth".

  - InfluxDB Details:

    - Organization: UNLV_Lab

    - Token: YOUR_API_TOKEN_HERE

    - Default Bucket: light_data

  - Click Save & Test.

- Create Dashboard:

  - + -> New Dashboard -> Add Visualization.

  - Select the InfluxDB source.

  - Paste this Flux query:

Code snippet

from(bucket: "light_data")
  |> range(start: -1h)
  |> filter(fn: (r) => r["_measurement"] == "apds9960_measurement")
  |> filter(fn: (r) => r["_field"] == "red" or r["_field"] == "blue" or r["_field"] == "green")
  |> aggregateWindow(every: 1s, fn: mean, createEmpty: false)
  |> yield(name: "mean")

  - Click Run Query. You should see the live RGB data lines.