### Phase 1: Raspberry Pi Zero W2 (Data Logger)

This script interacts directly with the APDS9960 sensor using the I2C bus, reads the light data, and saves it to a local SQLite database optimized for the Pi Zero's SD card.

#### Step 1: Hardware Setup

Connect the APDS9960 to the RPi Zero W2 GPIO header:

- VIN -> 3.3V (Pin 1)

- GND -> GND (Pin 6)

- SDA -> GPIO 2 (Pin 3)

- SCL -> GPIO 3 (Pin 5)

#### Step 2: Software Prerequisites

On your Raspberry Pi terminal, enable I2C and install the Python SMBus library:

Bash

# 1. Enable I2C interface

sudo raspi-config

# Navigate to: Interface Options -> I2C -> Yes -> Finish

# 2. Install I2C tools and Python SMBus

sudo apt update

sudo apt install python3-smbus i2c-tools

### Phase 3: Host Visualization (Mac/Windows)

This Python script uses Dash (by Plotly) to create a web-based dashboard. It reads the SQLite database and displays the Red, Green, Blue, and Clear values over time.

#### Step 1: Install Libraries

On your Mac or Windows PC:

Bash

#### pip install dash pandas plotly paramiko

#### Step 2: Run RPI Code On Raspberry Pi:

- Run python3 rpi_apds9960_sqlite_logger.py.

- It will verify the database is created at /home/$username/sensor_data.db and start printing log messages.

#### Step 3: Run the Dashboard

- Open your terminal/command prompt.

- Run python host_apds9960_sqlite_dash.py.

- Open your web browser and go to http://127.0.0.1:8050/.

- You will see the graph.